Portable Spirometer

- FVC, SVC and MVV tests

- Pre & Post medication results

- Indian Predicted equations

- Internal memory for 2500 tests

- COPD severity

- Overlapped FVC graph

- Auto-Interpretation

- Result table with Predicted, Result achieved, %Predicted, %Improvement

- Full Alphanumeric keypad to enter patient and clinic information

- Detachable and easy to disinfect digital turbine transducer

- Paper-out indication

- Battery status indicators

- PC connectivity for further viewing, reporting and management of Spirometry data

Technical Specifications

Spirometer

Flow Range              	-10 to +16 litres / sec.

Max Volume		8 litres

Accuracy		±1% (using a standard 3L calibration syringe)

Transducer		Bi-directional Turbine Cartridge

Turbine sensor  		Infrared interruption

Flow detection 		Volume differential

Display

Data display		Graphs and measurement values

Graphics display	320 x 240 pixels; TFT colored screen

Display size		3.5”

Printing

Recorder		Thermal printer

Size			80mm (Rolled)

Printing Labels 		Patient information, Clinic information

Printout format		Graphs and measurement values

Power Supply

Line Power		220 - 240V, 50Hz + 10%

Power Supply Operation AC/DC with on / off line Battery charging

Battery			 11.1V / 2600 mAh, Li-ion rechargeable

Charging Time		3 Hours

Mechanical

Dimensions 		288 mm x 238 mm x 85 mm

Weight			1.8 Kg approx.

PC Mode

Communication Interface USB

Features	PC Based Spirometry Test, Data download facility, Pediatric Incentives, Lung Age Calculation, Multiple Report Formats, Ethnic Correction, Unlimited storage